Assignment 1:						Subject: MPCA

Chapter 1: Overview of Computer and Microprocessor

- Differentiate between Microprocessor and Microcontroller.

- Differentiate between Von-Neumann and Harvard Architecture with suitable block diagrams.

- Explain the phases in instruction cycle.

- What are the applications of Microprocessor? Briefly explain.

- What are the applications of Microcontroller? Briefly explain.

- Explain the basic computer organization.

- Explain the linear organization of 8x2 ROM chip.

- Explain the horizontal organization of 8x2 ROM chip.

- Construct an 8x4 ROM from two 8x2 ROMs.  Also explain it.

- Construct an 8x4 ROM from two 8x2 ROMs using high-order interleaving. Also explain it.

- Construct an 8x4 ROM from two 8x2 ROMs using low-order interleaving. Also explain it.

- Design a 16x4 memory subsystem with high order interleaving using 8x2 memory chips for a computer system with an 8-bit address bus.

- Design a 16x4 memory subsystem with low order interleaving using 8x2 memory chips for a computer system with an 8-bit address bus.

- Design an interface for an input device which has binary address 1010 1010. Its computer system uses isolated I/0.

- Design an interface for an input device which has binary address 1010 1010. Its computer system uses memory mapped I/O.

- Design an interface for a bidirectional Input/output device that has binary address 1000 0001. Its computer system uses Isolated I/O.

- A CPU with 8-bit data bus and 8-bit address bus uses memory mapped I/O. It has 32 byte of ROM at 00H and 32 byte of ROM at 20H. It also has an I/O device at FFH. Show the load logic and enable logic.

- A computer system with an 8-bit address and an 8-bit data bus using isolated I/O. It has 16 x 8 ROM starting at address 00H constructed using 8 x8 chips; 64 x 8 of RAM starting at address 80H constructed using 64 x 4 chips. There is an I/O device at 40H. Show the design for the system.

- 8. A computer system with 8-bit data bus and 8-bit control bus uses isolated I/O. It has 64 bytes of ROM starting at 00H constructed using 64 x 4 chips; 128 bytes of RAM, starting at address 40H constructed using 32 x 8 chips; an input device with READY signal at address 40H; and an output with no READY signal at address 80H. Show the design for this system. Include all enable and load logic.

- A computer has a CPU with 8-bit address bus and 16-bit data bus. The computer uses Isolated I/O. It has 64 x 16 of ROM at 00 H; constructed using two 32 x 16 ROM chips. It also has 32 x16 of RAM at C0 H. The system has an input device at 17 H and an output device at B5 H. Show the design for the system including all the required logic. [PU 2018 Fall]

- A computer system with an 8-bit address bus and an 8-bit data bus uses memory-mapped I/O. It has 32 bytes of ROM starting at address 00H constructed using a single 32x8 chip; 128 bytes of RAM starting at address 80H constructed using 64x4 chips; a bidirectional I/O device with no READY signal at address 40H; and an input device with no READY signal at address 60H. Show the design for this system. Include all enable and load logic.

Chapter 4:  CPU design and Chapter 5:  Micro-sequencer Control Unit Design

- Design a CPU for below:

- Design the hardwired CU for question no 16

- Design a microsequencer control unit with horizontal microcode for the CPU described in question no. 16

- Design a microsequencer control unit with vertical microcode for the CPU described in question no. 16

- Design a microsequencer control unit which directly generate control signals for the CPU described in question no. 16

- There is a very simple CPU for the given set of Instruction:

Let the instruction width be 8 bits and address is 6 bits. Design the CPU’s register section, state diagram and ALU

- Design the hardwired control unit for the CPU described in above question.

- Design a microsequencer control unit with horizontal microcode for the CPU described in question no. 21

- Design a microsequencer control unit with vertical microcode for the CPU described in question no. 21

- Design a microsequencer control unit which directly generate control signals for the CPU described in question no. 21

Chapter 6:  Arithmetic Unit

- Trace RTL code to multiply 3x5 using shift add algorithm.

- Trace RTL code to multiply -7 x -9 using Booth’s algorithm.

Chapter 7:  Memory Organization

- Explain memory hierarchy.

- Explain Associative memory with its block diagram.

- Explain different cache mapping techniques.

Assignment 2:						Subject: MPCA

Chapter 2: 8085 MP

- Explain 8085 Microprocessor with its block diagram.

- What are accumulator and PC?

- Explain the 8085 pin configuration in brief.

- Define instruction. Classify 8085 instruction set and discuss in brief.

- Explain the address modes in 8085.

- What are instruction cycle, machine cycle and T-state and Timing diagram?

- Explain different types of machine cycle.

- Explain opcode fetch machine cycle with its timing diagram.

- Explain memory read/write machine cycle with its timing diagram.

- Explain IO read/write machine cycle with its timing diagram.

- Draw the timing diagram for MOV A,B

- Draw the timing diagram for MVI A, 43H

- Draw the timing diagram for STA 2050H

- Write an ALP for subtracting two 8-bit numbers stored in 2501H and 2502H. The result is to be stored in 2503H.

- Write an assembly language program to add two 8-bit numbers (F1 and 15), stored at C050H and C051H, with carry.

- Write a program in Assembly language to interchange(Swap) the contents of two memory location 2100H and 2101H.

- Define RTL. Consider a system with four 1-bit flipflops. Given RTL code where j, o, h and n are conditions which are mutually exclusive. Draw its hardware implementation.

- What are the uses of 8085 MP? Mention the issues in 8085.

Chapter 3: 8086 MP

- Explain the 8086 MP architecture with its block diagram.

- Explain the pin configuration of 8086 in brief.

- Explain different flags used in 8086 MP.

- Explain different General-purpose registers in 8086 MP.

- Explain different segment registers in 8086 MP.

- Explain the addressing modes of 8086 MP.

- Compare 8085 and 8086 MPs.

- What is editor, assembler and linker?

Chapter 7: IO organization

- Differentiate between Serial and Parallel Communication.

- Differentiate between Serial Interface and Parallel Interface.

- What is IO operation? Explain in brief about the different modes of IO transfer.

- Explain Programmed IO with a diagram.

- Explain Interrupt-driven IO.

- Explain DMA technique.

- What are the different types of interrupts? Explain in short.

- How interrupt processing is done? Explain.

- Explain priority arbitration with block diagram.

- Explain daisy chain arbitration with block diagram.

- What are IO processors? Explain.

- Describe UART.

Chapter 9: Advanced Architecture.

- Differentiate between RISC and CISC.

- Explain RISC instruction pipelining with different stages.

- What is register window? In a RISC processor system, there are 10 global registers and 8 register windows, each having 4 common input registers, 10 local registers, and 4 common output registers. How many registers are in this CPU? Also show the diagrammatic representation.

- Explain Flynn’s classification for multiprocessors.

- Explain different topologies for connection of multiprocessors.

- Explain UMA and NUMA.

- How can memory be accessed or shared in multiprocessors architecture? Explain each of them.

- What is cache coherence? Explain.

- Introduce multicore architecture with necessary diagrams.